NAME:BAVADARANI S (1912260562)

COURSE NAME;DIGITAL SIGNAL PROCESSING(ECA0902)

8. Realization of FIR Filters Using Direct, Cascade and Linear-Phase Structures Using

MATLAB

CODING;

clc;

clear;

close all;

% FIR coefficients

b = [1 2 3 3 2 1]/12;

Fs = 1000;

L = 128;

% Input impulse

x = [1 zeros(1,L-1)];

% Direct realization

y1 = filter(b,1,x);

% Cascade realization using two FIR sections

b1 = [1 1]/2;

b2 = [1 2 1]/6;

b3 = [1 1]/1;

% Use exact factorization of the original filter

% b = [1 2 3 3 2 1]/12

% Factor sections: (1+z^-1)(1+2z^-1+z^-2)(1+z^-1)

c1 = conv(b1,b2);

c2 = conv(c1,b3);

y2 = filter(c2,1,x);

% Frequency and phase response

[H,f] = freqz(b,1,512,Fs);

% Display results

disp('Direct output:');

disp(y1(1:6));

disp('Cascade output:');

disp(y2(1:6));

disp('Maximum error:');

disp(max(abs(y1-y2)));

% Figure 1: Direct and cascade outputs

figure(1);

subplot(2,1,1);

stem(0:L-1,y1);

title('Direct Realization');

xlabel('Sample');

ylabel('Amplitude');

grid on;

subplot(2,1,2);

stem(0:L-1,y2);

title('Cascade Realization');

xlabel('Sample');

ylabel('Amplitude');

grid on;

% Figure 2: Magnitude response

figure(2);

plot(f,20*log10(abs(H)+1e-8));

title('FIR Magnitude Response');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

grid on;

% Figure 3: Phase response

figure(3);

plot(f,unwrap(angle(H)));

title('FIR Phase Response');

xlabel('Frequency (Hz)');

ylabel('Phase (Radians)');

grid on;

% Symmetry check

disp('Symmetric coefficients (1 = Yes):');

disp(isequal(b,fliplr(b)));