NAME:BAVADARANI S (1912260562)

COURSE NAME;DIGITAL SIGNAL PROCESSING(ECA0902)

7.Design and Analysis of FIR Filters Using Parks–McClellan and Frequency-Sampling Techniques Using MATLAB

CODING;

clc; clear; close all;

Fs = 1000;

N = 20;

F = [0 100 150 500]/(Fs/2);

A = [1 1 0 0];

W = [1 10];

% Parks-McClellan method

b1 = firpm(N,F,A,W);

% Frequency-Sampling method

b2 = fir2(N,[0 100 150 500]/(Fs/2),A);

% Frequency responses

[H1,f] = freqz(b1,1,1024,Fs);

H2 = freqz(b2,1,1024,Fs);

% Plot magnitude responses

plot(f,abs(H1),'b',f,abs(H2),'r--','LineWidth',1.2);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude');

title('FIR Filter Design Comparison');

legend('Parks-McClellan','Frequency Sampling');

% Display coefficients

disp('Parks-McClellan coefficients:');

disp(b1);

disp('Frequency-Sampling coefficients:');

disp(b2);

% Passband ripple and stopband attenuation

pb = f <= 100;

sb = f >= 150;

fprintf('Parks-McClellan passband ripple: %.4f\n', ...

max(abs(abs(H1(pb))-1)));

fprintf('Parks-McClellan stopband attenuation: %.2f dB\n', ...

-20*log10(max(abs(H1(sb)))+eps))