Course name and code:DIGITAL SIGNAL PROCESSING ECA0902

Name:BAVADARANI S

Reg no:1912260562

SIMULATION EXPERIMENT – 5  Realization of IIR Filters Using Direct Form-I, Direct Form-II, Cascade and Parallel Structures Using MATLAB

CODING;

clc;

clear;

close all;

% Realization of IIR Filters Using Direct Form-I, Direct Form-II,

% Cascade and Parallel Structures

% Butterworth Low-Pass Filter Specifications

Fs = 1000;

Fc = 100;

N = 4;

% Design Butterworth IIR Low-Pass Filter

[b, a] = butter(N, Fc/(Fs/2), 'low');

disp('IIR Filter Numerator Coefficients (b):');

disp(b);

disp('IIR Filter Denominator Coefficients (a):');

disp(a);

% Generate Unit Impulse Input

x = [1 zeros(1,99)];

% Direct Form-I

y1 = filter(b, a, x);

disp('Direct Form-I Output:');

disp(y1(1:10));

% Direct Form-II

[sos, g] = tf2sos(b, a);

y2 = g * x;

for k = 1:size(sos,1)

y2 = filter(sos(k,1:3), sos(k,4:6), y2);

end

disp('Direct Form-II Output:');

disp(y2(1:10));

% Cascade Realization Using Second-Order Sections

y3 = g * x;

for k = 1:size(sos,1)

y3 = filter(sos(k,1:3), sos(k,4:6), y3);

end

disp('Cascade SOS Coefficients:');

disp(sos);

disp('Cascade Output:');

disp(y3(1:10));

% Parallel Realization Using Partial Fraction Expansion

[r, p, k] = residuez(b, a);

disp('Parallel Branch Residues:');

disp(r);

disp('Parallel Branch Poles:');

disp(p);

disp('Direct Term Coefficients:');

disp(k);

% Construct Parallel Realization

y4 = zeros(size(x));

if ~isempty(k)

y4(1) = k(1) * x(1);

for n = 2:length(x)

if n <= length(k)

y4(n) = k(n);

end

end

end

for i = 1:length(r)

branch_b = [r(i), 0];

branch_a = [1, -p(i)];

y4 = y4 + filter(branch_b, branch_a, x);

end

% Compare Responses

error_df2 = max(abs(y1 - y2));

error_cascade = max(abs(y1 - y3));

error_parallel = max(abs(y1 - y4));

disp('Maximum Error - Direct Form-II:');

disp(error_df2);

disp('Maximum Error - Cascade:');

disp(error_cascade);

disp('Maximum Error - Parallel:');

disp(error_parallel);

% Plot Impulse Responses

figure;

subplot(2,2,1);

stem(y1, 'filled');

title('Direct Form-I');

xlabel('Samples');

ylabel('Amplitude');

grid on;

subplot(2,2,2);

stem(y2, 'filled');

title('Direct Form-II');

xlabel('Samples');

ylabel('Amplitude');

grid on;

subplot(2,2,3);

stem(y3, 'filled');

title('Cascade Realization');

xlabel('Samples');

ylabel('Amplitude');

grid on;

subplot(2,2,4);

stem(y4, 'filled');

title('Parallel Realization');

xlabel('Samples');

ylabel('Amplitude');

grid on;

% Combined Response Comparison

figure;

plot(y1, 'b', 'LineWidth', 1.5);

hold on;

plot(y2, 'r--');

plot(y3, 'g:');

plot(y4, 'k-.');

title('Comparison of IIR Realization Structures');

xlabel('Samples');

ylabel('Amplitude');

legend('Direct Form-I', 'Direct Form-II', ...

'Cascade', 'Parallel');

grid on;

hold off;

disp('IIR Filter Realization Completed Successfully.');