Course name and code:DIGITAL SIGNAL PROCESSING ECA0902

Name:BAVADARANI S

Reg no:1912260562

SIMULATION EXPERIMENT – 3

Design of IIR Filters Using Impulse Invariance and Bilinear Transformation Methods

Using MATLAB

CODING;

clc;

clear;

close all;

% Simulation Experiment 3

% Design of IIR Filters Using Impulse Invariance

% and Bilinear Transformation

%% 1. Input Specifications

Fs = 1000;              % Sampling frequency (Hz)

T = 1/Fs;               % Sampling period

Fp = 200;               % Passband edge (Hz)

Fst = 300;              % Stopband edge (Hz)

Ap = 1;                 % Passband attenuation (dB)

As = 40;                % Stopband attenuation (dB)

Wp = 2*pi*Fp;           % Analog passband edge (rad/s)

Ws = 2*pi*Fst;          % Analog stopband edge (rad/s)

fprintf('Sampling Period T = %f seconds\n', T);

%% 2. Design Analog Butterworth Filter

% Use prewarping for bilinear transformation

wp = 2*Fs*tan(pi*Fp/Fs);

ws = 2*Fs*tan(pi*Fst/Fs);

[N, Wn] = buttord(wp, ws, Ap, As, 's');

[bA, aA] = butter(N, Wn, 's');

fprintf('\nAnalog Butterworth Filter Order = %d\n', N);

%% 3. Impulse Invariance Method

[bII, aII] = impinvar(bA, aA, Fs);

fprintf('\nImpulse Invariance Coefficients:\n');

disp('Numerator bII = ');

disp(bII);

disp('Denominator aII = ');

disp(aII);

%% 4. Bilinear Transformation Method

[bBL, aBL] = bilinear(bA, aA, Fs);

fprintf('\nBilinear Transformation Coefficients:\n');

disp('Numerator bBL = ');

disp(bBL);

disp('Denominator aBL = ');

disp(aBL);

%% 5. Frequency Response Comparison

[HII, f] = freqz(bII, aII, 2048, Fs);

[HBL, ~] = freqz(bBL, aBL, 2048, Fs);

figure;

subplot(2,1,1);

plot(f, 20*log10(abs(HII)+eps), 'b', 'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(HBL)+eps), 'r--', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response Comparison');

legend('Impulse Invariance', 'Bilinear Transformation');

xlim([0 Fs/2]);

ylim([-100 5]);

subplot(2,1,2);

plot(f, unwrap(angle(HII)), 'b', 'LineWidth', 1.5);

hold on;

plot(f, unwrap(angle(HBL)), 'r--', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (radians)');

title('Phase Response Comparison');

legend('Impulse Invariance', 'Bilinear Transformation');

xlim([0 Fs/2]);

%% 6. Bilinear Transformation Without Prewarping

[bBL0, aBL0] = bilinear(bA, aA, Fs);

figure;

freqz(bBL0, aBL0, 2048, Fs);

title('Bilinear Transformation Frequency Response');

%% 7. Cutoff Frequency Estimation

magII = 20*log10(abs(HII)+eps);

magBL = 20*log10(abs(HBL)+eps);

[~, idxII] = min(abs(magII + 3));

[~, idxBL] = min(abs(magBL + 3));

fprintf('\nApproximate -3 dB Frequencies:\n');

fprintf('Impulse Invariance = %.2f Hz\n', f(idxII));

fprintf('Bilinear Transformation = %.2f Hz\n', f(idxBL));

%% 8. Display Analog and Digital Filter Responses

figure;

wa = linspace(0, pi*Fs, 2048);

Ha = freqs(bA, aA, wa);

plot(wa/(2*pi), 20*log10(abs(Ha)+eps), 'k', ...

'LineWidth', 1.5);

hold on;

plot(f, magII, 'b--', 'LineWidth', 1.5);

plot(f, magBL, 'r-.', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Analog and Digital Filter Responses');

legend('Analog Prototype', 'Impulse Invariance', ...

'Bilinear Transformation');

xlim([0 Fs/2]);

ylim([-100 5]);

%% 9. Final Result

disp(' ');

disp('Experiment completed successfully.');

disp('Compare the coefficients and frequency responses');

disp('to study aliasing and frequency warping.');