NAME:BAVADARANI S (1912260562)

COURSE NAME;DIGITAL SIGNAL PROCESSING(ECA0902)

SIMULATION EXPERIMENT –6  Design and Analysis of FIR Digital Filters Using Fourier Series and Windowing Techniques Using MATLAB

CODING;

clc;

clear;

close all;

Fs = 1000;

Fp = 100;

Fst = 150;

N = 20;

Fc = (Fp + Fst)/2;

Wn = Fc/(Fs/2);

b1 = fir1(N, Wn, 'low', rectwin(N+1));

b2 = fir1(N, Wn, 'low', hamming(N+1));

b3 = fir1(N, Wn, 'low', hann(N+1));

b4 = fir1(N, Wn, 'low', blackman(N+1));

disp('Rectangular coefficients:');

disp(b1);

disp('Hamming coefficients:');

disp(b2);

disp('Hanning coefficients:');

disp(b3);

disp('Blackman coefficients:');

disp(b4);

[H1, f] = freqz(b1, 1, 1024, Fs);

[H2, ~] = freqz(b2, 1, 1024, Fs);

[H3, ~] = freqz(b3, 1, 1024, Fs);

[H4, ~] = freqz(b4, 1, 1024, Fs);

figure;

plot(f, 20*log10(abs(H1)+1e-8), 'r');

hold on;

plot(f, 20*log10(abs(H2)+1e-8), 'b');

plot(f, 20*log10(abs(H3)+1e-8), 'g');

plot(f, 20*log10(abs(H4)+1e-8), 'k');

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Magnitude Response of FIR Filters');

legend('Rectangular','Hamming','Hanning','Blackman');

figure;

plot(f, unwrap(angle(H1)), 'r');

hold on;

plot(f, unwrap(angle(H2)), 'b');

plot(f, unwrap(angle(H3)), 'g');

plot(f, unwrap(angle(H4)), 'k');

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (radians)');

title('Phase Response of FIR Filters');

legend('Rectangular','Hamming','Hanning','Blackman');