Course name and code:DIGITAL SIGNAL PROCEESING ECA0902

Name:BAVADARANI S

Reg no:1912260562

2. Design and Performance Analysis of Chebyshev Type-I IIR Low-Pass and High-Pass Filters Using MATLAB

C clc;

clear;

close all;

%% Given Specifications

Fs  = 1000;     % Sampling frequency in Hz

Fp  = 100;      % Passband edge in Hz

Fst = 150;      % Stopband edge in Hz

Rp  = 1;        % Passband ripple in dB

Rs  = 40;       % Stopband attenuation in dB

%% Nyquist frequency

Fn = Fs/2;

%% Normalized frequencies

Wp = Fp/Fn;

Ws = Fst/Fn;

fprintf('Normalized Passband Frequency = %.2f\n', Wp);

fprintf('Normalized Stopband Frequency = %.2f\n', Ws);

%% ---------------------------------------------------------

% CHEBYSHEV TYPE-I LOW-PASS FILTER

% ----------------------------------------------------------

[Nc_lp, Wc_lp] = cheb1ord(Wp, Ws, Rp, Rs);

[b_cheb_lp, a_cheb_lp] = cheby1(Nc_lp, Rp, Wc_lp, 'low');

fprintf('\nCHEBYSHEV TYPE-I LOW-PASS FILTER\n');

fprintf('Filter Order = %d\n', Nc_lp);

fprintf('Cutoff Frequency = %.4f (normalized)\n', Wc_lp);

disp('Numerator coefficients b = ');

disp(b_cheb_lp);

disp('Denominator coefficients a = ');

disp(a_cheb_lp);

%% Frequency response

[Hc_lp, f] = freqz(b_cheb_lp, a_cheb_lp, 2048, Fs);

%% ---------------------------------------------------------

% CHEBYSHEV TYPE-I HIGH-PASS FILTER

% ----------------------------------------------------------

% For high-pass filter, passband is above Fp

% and stopband is below Fst.

Wp_hp = Fst/Fn;

Ws_hp = Fp/Fn;

[Nc_hp, Wc_hp] = cheb1ord(Wp_hp, Ws_hp, Rp, Rs);

[b_cheb_hp, a_cheb_hp] = cheby1(Nc_hp, Rp, Wc_hp, 'high');

fprintf('\nCHEBYSHEV TYPE-I HIGH-PASS FILTER\n');

fprintf('Filter Order = %d\n', Nc_hp);

fprintf('Cutoff Frequency = %.4f (normalized)\n', Wc_hp);

disp('Numerator coefficients b = ');

disp(b_cheb_hp);

disp('Denominator coefficients a = ');

disp(a_cheb_hp);

%% Frequency response

[Hc_hp, f] = freqz(b_cheb_hp, a_cheb_hp, 2048, Fs);

%% ---------------------------------------------------------

% BUTTERWORTH LOW-PASS FILTER

% ----------------------------------------------------------

[Nb_lp, Wb_lp] = buttord(Wp, Ws, Rp, Rs);

[b_butt_lp, a_butt_lp] = butter(Nb_lp, Wb_lp, 'low');

fprintf('\nBUTTERWORTH LOW-PASS FILTER\n');

fprintf('Filter Order = %d\n', Nb_lp);

fprintf('Cutoff Frequency = %.4f (normalized)\n', Wb_lp);

%% Frequency response

[Hb_lp, ~] = freqz(b_butt_lp, a_butt_lp, 2048, Fs);

%% ---------------------------------------------------------

% BUTTERWORTH HIGH-PASS FILTER

% ----------------------------------------------------------

[Nb_hp, Wb_hp] = buttord(Wp_hp, Ws_hp, Rp, Rs);

[b_butt_hp, a_butt_hp] = butter(Nb_hp, Wb_hp, 'high');

fprintf('\nBUTTERWORTH HIGH-PASS FILTER\n');

fprintf('Filter Order = %d\n', Nb_hp);

fprintf('Cutoff Frequency = %.4f (normalized)\n', Wb_hp);

%% Frequency response

[Hb_hp, ~] = freqz(b_butt_hp, a_butt_hp, 2048, Fs);

%% ---------------------------------------------------------

% MAGNITUDE RESPONSE - LOW PASS

% ----------------------------------------------------------

figure;

plot(f, 20*log10(abs(Hc_lp)), 'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(Hb_lp)), '--', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Chebyshev-I vs Butterworth Low-Pass Filters');

legend('Chebyshev-I', 'Butterworth');

xlim([0 300]);

ylim([-100 5]);

xline(Fp, ':k', 'Passband Edge');

xline(Fst, ':k', 'Stopband Edge');

%% ---------------------------------------------------------

% MAGNITUDE RESPONSE - HIGH PASS

% ----------------------------------------------------------

figure;

plot(f, 20*log10(abs(Hc_hp)), 'LineWidth', 1.5);

hold on;

plot(f, 20*log10(abs(Hb_hp)), '--', 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Chebyshev-I vs Butterworth High-Pass Filters');

legend('Chebyshev-I', 'Butterworth');

xlim([0 500]);

ylim([-100 5]);

xline(Fp, ':k', 'Stopband Edge');

xline(Fst, ':k', 'Passband Edge');

%% ---------------------------------------------------------

% PHASE RESPONSE - CHEBYSHEV LOW PASS

% ----------------------------------------------------------

figure;

plot(f, unwrap(angle(Hc_lp)), 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (radians)');

title('Phase Response of Chebyshev-I Low-Pass Filter');

xlim([0 300]);

%% ---------------------------------------------------------

% PHASE RESPONSE - CHEBYSHEV HIGH PASS

% ----------------------------------------------------------

figure;

plot(f, unwrap(angle(Hc_hp)), 'LineWidth', 1.5);

grid on;

xlabel('Frequency (Hz)');

ylabel('Phase (radians)');

title('Phase Response of Chebyshev-I High-Pass Filter');

xlim([0 500]);

%% ---------------------------------------------------------

% PASSBAND RIPPLE VERIFICATION - LOW PASS

% ----------------------------------------------------------

passband_lp = f <= Fp;

mag_lp_db = 20*log10(abs(Hc_lp));

max_pass_lp = max(mag_lp_db(passband_lp));

min_pass_lp = min(mag_lp_db(passband_lp));

fprintf('\nLOW-PASS PASSBAND ANALYSIS\n');

fprintf('Maximum passband magnitude = %.4f dB\n', max_pass_lp);

fprintf('Minimum passband magnitude = %.4f dB\n', min_pass_lp);

fprintf('Passband ripple = %.4f dB\n', ...

max_pass_lp - min_pass_lp);

%% ---------------------------------------------------------

% STOPBAND ATTENUATION - LOW PASS

% ----------------------------------------------------------

stopband_lp = f >= Fst;

max_stop_lp = max(mag_lp_db(stopband_lp));

fprintf('Maximum stopband magnitude = %.4f dB\n', max_stop_lp);

fprintf('Minimum stopband attenuation = %.4f dB\n', -max_stop_lp);

%% ---------------------------------------------------------

% PASSBAND ANALYSIS - HIGH PASS

% ----------------------------------------------------------

passband_hp = f >= Fst;

mag_hp_db = 20*log10(abs(Hc_hp));

max_pass_hp = max(mag_hp_db(passband_hp));

min_pass_hp = min(mag_hp_db(passband_hp));

fprintf('\nHIGH-PASS PASSBAND ANALYSIS\n');

fprintf('Maximum passband magnitude = %.4f dB\n', max_pass_hp);

fprintf('Minimum passband magnitude = %.4f dB\n', min_pass_hp);

fprintf('Passband ripple = %.4f dB\n', ...

max_pass_hp - min_pass_hp);

%% ---------------------------------------------------------

% STOPBAND ATTENUATION - HIGH PASS

% ----------------------------------------------------------

stopband_hp = f <= Fp;

max_stop_hp = max(mag_hp_db(stopband_hp));

fprintf('Maximum stopband magnitude = %.4f dB\n', max_stop_hp);

fprintf('Minimum stopband attenuation = %.4f dB\n', -max_stop_hp);

%% ---------------------------------------------------------

% ORDER COMPARISON

% ----------------------------------------------------------

fprintf('\n========================================\n');

fprintf('FILTER ORDER COMPARISON\n');

fprintf('========================================\n');

fprintf('Chebyshev-I Low Pass  = %d\n', Nc_lp);

fprintf('Butterworth Low Pass  = %d\n', Nb_lp);

fprintf('Chebyshev-I High Pass = %d\n', Nc_hp);

fprintf('Butterworth High Pass = %d\n', Nb_hp);